# Enterprise Campus Network – IP Addressing & Subnetting Plan

This document provides a structured IP addressing and subnetting plan derived from the provided campus network topology. The goal is to ensure scalability, clear segmentation, and professional documentation suitable for network engineering portfolios and recruiter review.

## 1. Network Overview

The topology consists of a Main Branch Network, Branch Campus Network, DMZ/Server segment, WAN links, and multiple VLAN‑based access networks. Subnetting is used to separate departments, management traffic, wireless users, and infrastructure.

## 2. Core Infrastructure Networks

| Network Segment | Subnet | Subnet Mask | Purpose |
| --- | --- | --- | --- |
| DMZ / Server Network | 10.20.20.0/27 | 255.255.255.224 | Public services (DNS, WEB, EMAIL, FTP, DHCP) |
| HQ Firewall to Core | 10.20.20.32/30 | 255.255.255.252 | Firewall to core routing |
| Core L3 Switch Link | 10.20.20.36/30 | 255.255.255.252 | Core switch interconnect |
| WAN Link Router0‑Router1 | 20.20.20.0/30 | 255.255.255.252 | WAN provider connection |
| WAN Link Router1‑Router2 | 30.30.30.0/30 | 255.255.255.252 | Branch WAN connection |
| ISP Transit | 105.100.50.0/30 | 255.255.255.252 | Internet edge transit |
| Branch Firewall WAN | 205.200.100.0/30 | 255.255.255.252 | Branch internet access |

## 3. VLAN Segmentation Plan

| VLAN | Department | Subnet | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| VLAN 10 | Management | 192.168.10.0/24 | 255.255.255.0 | 192.168.10.1 |
| VLAN 20 | Health Sciences | 172.16.10.0/24 | 255.255.255.0 | 172.16.10.1 |
| VLAN 30 | Business | 172.16.20.0/24 | 255.255.255.0 | 172.16.20.1 |
| VLAN 40 | Engineering | 172.16.30.0/24 | 255.255.255.0 | 172.16.30.1 |
| VLAN 50 | Arts | 172.16.40.0/24 | 255.255.255.0 | 172.16.40.1 |
| VLAN 60 | Branch Health Sciences | 172.17.10.0/24 | 255.255.255.0 | 172.17.10.1 |
| VLAN 70 | Branch Business | 172.17.20.0/24 | 255.255.255.0 | 172.17.20.1 |
| VLAN 80 | Branch Engineering | 172.17.30.0/24 | 255.255.255.0 | 172.17.30.1 |
| VLAN 90 | Wireless LAN | 10.10.0.0/16 | 255.255.0.0 | 10.10.0.1 |
| VLAN 199 | Blackhole | Null Route | N/A | Used for unused VLAN shutdown |

## 4. IP Allocation Strategy

• /30 subnets are used for point‑to‑point WAN links to minimize address waste.
• /27 subnet is used in the DMZ to allow multiple public‑facing servers.
• /24 networks are used for departmental VLANs to simplify management and scaling.
• Private RFC1918 addressing ensures internal routing security.
• VLAN segmentation reduces broadcast domains and improves security.

## 5. Benefits of this Design

• Clear separation of departments
• Efficient WAN addressing
• Secure DMZ isolation
• Scalable addressing for future growth